Fault diagnosis for discrete monitoring data based on fusion algorithm

He Sijie, Peng Yu, Liu Datong,

Department of Automatic Test and Control, Harbin Institute of Technology

Harbin 150080, China

Phone:       , Fax:         , Email:

Abstract –Fault diagnosis has a significant role in enhancing the safety, reliability, and availability of complex systems. However, the problem of enormous condition monitoring data and multiple failure modes makes the diagnostics great challenge. The imbalance between normal and fault monitoring data will increase the false alarm rate and the false negative rate. On the other hand, discrete monitoring data such as events are frequent and critical to fault diagnosis of complex systems. In this work, we propose a fusion fault diagnostic method which combines Naïve Bayes with AdaBoost ensemble algorithm. This integrated method is appropriate for discrete data and improves the adaptability for imbalanced condition monitoring data. Experimental results based on PHM 2013 dataset show that fault diagnosis performance using the fusion method can be ameliorated.

Keywords – Fault diagnosis, Naïve Bayes, AdaBoost, discrete monitoring data, fusion algorithm.

- INTRODUCTION

As the system complexity increases rapidly in aerospace engineering, the requirements of reliability and safety become much higher. However, for most space crafts, because of the high complexity of space environment and the limitation of measurements, abnormal operating conditions even the system failures appear. Therefore the remote condition monitoring data become the main basis for fault diagnosis [1-2]. Thus, data-driven fault diagnosis has gradually become the focus for improving the reliability, safety and availability for the aerospace applications [3-4].

Fault diagnosis based on data-driven methods has been extensively studied, which applies historical monitoring data to learn a model of system behavior [5]. Such approaches do not rely on the detailed mathematical model of complex systems. And in hybrid complex systems, of which behavior includes both continuous and discrete monitoring data, achieving accurate diagnosis becomes more difficult. Discrete monitoring data containing a large amount of systematic information are practical and adaptable for fault diagnosis, applied solely [6]. Hence, the fault diagnosis of discrete monitoring data based on fusion algorithm is concerned in this paper.

Data-driven diagnostic approaches are mostly based on machine learning algorithm. Lots of contributions have been made in this area. The algorithms, such as Support Vector Machine [7], Naïve Bayes [8], Decision Tree [9] and Neural Network [10], are applied for fault diagnosis. To improve the diagnostic accuracy, boosting algorithms like AdaBoost [11-12], are used to stabilize the single algorithm. Due to discrete feature and imbalance between normal and fault conditions, in this work, the fusion method combining Naïve Bayes and AdaBoost ensemble algorithm is proposed.

This paper is organized as follows. In section II, preliminary work including the Naïve Bayes and AdaBoost ensemble algorithm will be introduced briefly. In section III, the fault diagnosis based on proposed fusion method is discussed in detail. In Section IV, experimental results are provided to validate the performance of the proposed approach. Finally, the conclusion and the future work will be presented.

- METHODOLOGY

- Naïve Bayes algorithm

As a type of machine learning algorithm, Naïve Bayes classifiers [13] is a simple probabilistic classifier based on Bayes' theorem. The main task is to approximate an unknown target function, or equivalently .

Assume that Y is a finite discrete random variable, and  is a n-dimension vector, where  is the ith attribute of X. Applying Bayes rule,  can be represented as

(1)

where  denotes the mth type as Y, and  is the attribute vector for kth sample. Training data are analyzed to estimate  and . The posterior probability for new instance  is determined with Bayes rules as accordingly.

Naive Bayes algorithm proposes conditionally independent assumption for attributes, which simplifies the representation of . Hypothetically, given classification as Y, the attributes  of one sample are conditionally independent of one another, which means

(2)

With Y as a discrete-valued variable and attributes  as discrete or real-valued variable, a classifier can be constructed to estimate the probability distribution of possible value of Y, for new testing sample X. According to Bayes theorem, the expression for the probability of Y, which is supposed to be kth possible value, is

(3)

where all possible j for Y are considered to calculate the denominator of (3). Due to conditionally independent assumption, we can rewrite (3) as

(4)

which is regarded as the most significant equation for Naive Bayes classifier. If a new instance  updates, the probability which Y will take on any possible value can be calculated according to (4) with the estimation of  and . Generally, the most likely value of Y is regarded as the final result, so the rule of classifier is

(5)

In practice, the corresponding frequency of different samples in the training data is applied to estimate the prior probability with maximum likelihood.

- AdaBoost ensemble algorithm

“Boosting” is an ensemble learning algorithm which improves the performance of classification algorithm by combining several classifiers to create a single predictive model [14]. Adaptive Boosting, abbreviated as AdaBoost, does not require any prior knowledge about the performance of the weak learning algorithm [15].

Assuming  is the training set with m samples and  is the class label of . In this paper, we consider labels Y as finite number k. In AdaBoost algorithm, a “weak” learning algorithm is necessary, which is denoted as weak classifier. AdaBoost algorithm constructs weak classifiers tautologically in several rounds. On round t, the distribution  of training data is generated by results of tth weak classifiers. And then, a new weak classifier is built and get back a hypothesis , which accurately classifies samples with high weights in distribution . Therefore, the task of weak classifier is to minimize the weighted error  of training set, where  is the weight decided by the results of previous weak classifier. Repeat such procedure for T times, and finally combine all weak classifiers with respect to different weights, so we can create a new classification model as the finial hypothesis .

In initialization, the distribution is uniform over the training set and the updated distribution  is computed from both distribution  and the results of former weak classifier. If the former weak classifier has classified the sample correctly, its weight is shrunken by a number , and otherwise the weight remains unchanged. The number  is the function of weighted error rate . Therefore samples which are classified accurately by most previous weak classifiers hold lower weights than those tending to be misclassified. AdaBoost algorithm focuses more on “hard” samples, which cannot be classified correct by former weak classifiers.

At last, the final hypothesis  is the combination of weak classifiers with a weighted vote. The weight of each weak classifier is defined as , thus, higher weight is given to the weak classifier with better performance. When a new instance x is given, the final classifier outputs a label whose sum of weak classifier’s results is the maximum.

The detailed procedure of AdaBoost is as follows.

Input: m training samples with labels ;

Integer T specifies number of iterations;

The distribution of training set .

Initialize the weight for all training samples.

Do for :

- Call weak classifier, providing it with distribution .

- Get back a hypothesis .

Calculate weighted error rate: .

- Set .

- Update distribution  by .

- Normalize the update distribution .

Output hypothesis .

- PROPOSED FUSION FRAMEWORK FOR FAULT DIAGNOSIS

To ameliorate the fault diagnosis using basic Naïve Bayes classifier for imbalance data between normal and fault conditions, we combine the AdaBoost ensemble algorithm to obtain a fusion framework. The detailed process is shown in Fig.1.

Fig. 1 Proposed framework of fault diagnosis based on fusion method

Step 1: Initialize data

To learn the distribution, a condition characteristic matrix is constructed to show the relationship between each type of faults and the frequency of all events, which is shown as (6) for modeling the fault diagnostic situation.

(6)

where  denotes the ith type of events,  demonstrates the jth condition with the sum as K, and  shows the occurrence number of ith event in jth condition.

By this time, the entire training samples are allocated the same weights as  to train the weak classifier.

Step 2: Train weak classifier

Step 2-1: Preprocess the weighted training set

To illuminate the connection between problem conditions, which are usually described as faults, a fault characteristic matrix is necessary as

(7)

where  is defined as the set of all possible faults and  is the label used for non-fault condition.

Combine the condition and fault characteristic matrixes together as the final matrix in (8), which is applied to build the weak classifier.

(8)

The element nij represents the occurrence number of certain event, which is classified as type i, when the fault Pj is met in the system.

Step 2-2: Calculate the posterior probability for each event.

Applying the information offered by the final matrix in step 2-1, we estimate the conditional probabilities  of observing a condition that includes the event E.

(9)

The occurrence number in the final matrix is substituted with the posterior probability. To avoid zero appears in the matrix which may influence the computational procedure, we use a little number tending to zero instead.

(10)

where is the probability for ith event appearing in jth problem.

Step 2-3: Calculate the posterior probability for each condition

Due to the conditional independence in Naïve Bayes algorithm, different events deriving from the same condition are independent with each other. So the posterior probability for one condition C, with m kinds of events, leading certain problem can be calculated as

(11)

where  is  from probabilistic matrix in step 2-2.

Step 2-4: Fault diagnosis

According to the Bayes rule, the posterior probability of certain condition for each fault can be determined as

(12)

Moreover, in this case we regard the probability of each condition as the same, therefore, only the posterior probability for certain condition  and the prior probability for each problem  are involved in this situation without considering the influence of denominator in (12). Generally, we recommend the fault condition with the largest product of the two factors as the real problem.

Step 3: Update the distribution

To evaluate the performance of the weak classifier, we compare the predicted results from our classifier with the true fault condition and calculate the weighted error rate e.

(13)

The weighted error rate adds up all weights which come from the conditions classified improperly. According to AdaBoost algorithm, number  is needed to adjust the distribution of training dataset.

(14)

And the updated weight is

(15)

where  indicates the updated weight for ith condition, that is, the weights of conditions classified inaccurately remain unchanged, while others decrease. In this case, the classifier would pay more attention to the conditions that cannot be classified correctly. Then we normalize the distribution of the training dataset, which means the sum of all weights become 1. So far the distribution of training dataset has already been updated based on the performance of the weak classifier.

And then, renovate the training dataset, where  for each condition is multiplied with its updated weight as

(16)

where  represents the occurrence number for jth event in ith condition. Adjust the whole training dataset in same way to obtain a new weighted training dataset for the construction of a new weak classifier.

Step 4: Repeat several weak classifiers

Repeat the Step 2 to Step 3, we can build several weak classifiers until the weighted accuracy is larger than 0.5, which implies the new classifier cannot improve the performance compared with the former weak classifier and cannot bring positive effect to the final classifier.

Step 5: Integrate the results of different weak classifiers

Assemble all weak classifiers together to produce the final results. The proportion of each classifier is determined by its performance.

(17)

where  is the predicted fault type for each condition and  is the weight of tth weak classifier.

When a new sample is provided, it should be classified by all weak classifiers and those results will be integrated to determine the final result with (17).

- EXPERIMENTAL RESULTS AND ANALYSIS

- PHM Data challenge dataset

The PHM data challenge dataset is presented at the 2013 PHM annual conference by NASA and PHM Society [16][17]. This dataset offers state monitoring data of a complex system, which includes a known set of faults appearing in historical data. The challenge is designed that a large number of unlabeled samples are provided. Whether these unlabeled samples are faults, described as problems in this dataset, or are normal states should be determined with the training of known samples from historical monitoring data. Also the specific type of such problem needs to be distinguished.

In PHM 2013 data challenge dataset, each candidate problem is represented as a case, which comprises a collection of event codes with observations of 30 parameters. Event codes were reported automatically from the monitoring system. When a particular condition happens, a corresponding event code will be generated. Under this circumstance, the engineers or the control system can mark such cases, which can also correspond to a single event. At the same time, once an event occurs, the monitoring system will take a snapshot for all 30 monitoring parameters.

In the experiments, we divide the dataset as training and testing set for evaluation. The dataset consists of cases determined as nuisances or problems in historical data, and the exact labels of such problems are provided. In this dataset, there are 10459 cases, of which 10295 cases are determined as nuisances and 164 as problems which are characterized as 13 different problem types. For all cases, 1,316,653 events codes are recorded with measurements of continuous parameters. Therefore, the task of the dataset is to discard nuisance cases and distinguish the correct problem types for objective cases.

- Evaluation criterion

In the experiments, we select Fault accuracy and Overall accuracy to evaluate the proposed method. Fault accuracy represents the diagnostic accuracy only for problem cases, while the Overall accuracy demonstrates the diagnostic accuracy for all cases.

(18)

(19)

- Results and discussion

The data set is of large scale, imbalance and high complexity. To evaluate the performance of the improved algorithm, we divide the dataset into training subset and testing subset, which are shown in Table I.

Table I. Division of training and testing dataset

| Label | Training set |  | Testing set |  |
| --- | --- | --- | --- | --- |
| Case Style | Problem | Nuisance | Problem | Nuisance |
| Case Account | 122 | 7222 | 42 | 3096 |
| Proportion | 7344 (70%) |  | 3138(30%) |  |

(1) Diagnostics with basic Naïve Bayes classifier

We first use the basic Naïve Bayes classifier to conduct fault diagnosis, in which the nuisance is disregarded. The quantitative results are shown in Table II.

Table II. Diagnostic results with basic Naïve Bayes algorithm

| In actual application, i.e., condition monitoring of space crafts and satellites, these complex system are always operating under normal operation in almost all the time. Thus, the monitoring data must be of high imbalance feature. Considering actual data feature and nuisance comprehensively, the diagnostic results are shown in Table III. Label | Training Set | Testing Set |
| --- | --- | --- |
| Correct Classification | 107 | 19 |
| Case Capacity | 122 | 42 |
| Accuracy | 87.70% | 45.20% |

Table III. Results of training and testing set on overall condition

| Label | Training Set |  | Testing Set |  |
| --- | --- | --- | --- | --- |
| Case Label | Problem | Nuisance | Problem | Nuisance |
| Case Capacity | 122 | 7222 | 42 | 3096 |
| Correct Classification | 97 | 6056 | 10 | 2566 |
| Individual Accuracy | 79.51% | 83.85% | 23.81% | 82.88% |
| Overall Accuracy | 83.67% |  | 81.65% |  |

From Table III, we can see that the performance of Naïve Bayes algorithm considering the nuisance is not satisfied. It indicates that the basic Naïve Bayes algorithm cannot meet the industrial requirements.

(2) Diagnostics with improved method

Then, we conduct the same diagnostic experiments with the proposed method, and the results are shown in Table IV.

Table IV. Diagnostic results with fusion method

| Label | Training set |  | Testing set |  |
| --- | --- | --- | --- | --- |
| Case Label | Problem | Nuisance | Problem | Nuisance |
| Case Capacity | 122 | 7222 | 42 | 3096 |
| Correct Classification | 102 | 6311 | 12 | 2691 |
| Individual Accuracy | 83.61% | 87.38% | 28.57% | 86.92% |
| Overall Accuracy | 87.32% |  | 86.13% |  |

By analyzing the results in above tables, we can see that:

1. The implementation of basic Naïve Bayes algorithm can achieve the faults diagnosis for problems in this dataset, with the overall accuracy above 80%.

2. The combination of the Naïve Bayes and AdaBoost ensemble algorithm can partly overcome data imbalance issue, and the fault accuracy increased nearly 5% as well.

The fusion method can achieve the fault diagnosis for discrete monitoring data in complex systems and adapts better for imbalance in frequency of fault and normal condition. As shown in experimental results, the proposed method achieves incensement of fault diagnostic accuracy without impairment in overall accuracy.

Compared with basic Naïve Bayes, the fusion method attaches more attention to the “hard” samples, which are difficult to be diagnosed correctly. The weights of such samples, which are usually fault conditions, are fortified. When several weak classifiers are trained, each of which focuses on “hard” samples from the former weak classifier model. Integrating all weak classifiers with different weights can comprehensively consider the feature of monitoring data to achieve proper fault diagnosis for both fault and normal conditions and improve the performance of basic Naïve Bayes algorithm.

- CONCLUSION AND FUTURE WORK

The contributions of this work can be concluded as below. (1) A data-driven fault diagnosis method based on discrete monitoring data is proposed. (2) A fusion classification method integrates Naïve Bayes classifier and AdaBoost ensemble algorithm to improve the diagnostic performance and overcome data imbalance between fault and normal samples. (3) The proposed diagnostic method is applied on PHM 2013 data challenge dataset and achieves higher fault diagnostic accuracy than basic Naïve Bayes classifier.

In future, more research work should continually focus on the improvement for fault diagnostic accuracy. The proposed method is only suitable for discrete monitoring data. However, in most of complex systems such as satellites and aircrafts, the monitoring data involves multiple types of parameters. Thus, a comprehensive method which can both handle the continuous and discrete parameters is essential for actual industrial applications.

ACKNOWLEDGMENT

This work is supported partly by National Natural Science Foundation of China under Grant No. 61301205, Research Fund for the Doctoral Program of Higher Education of China under Grant No. 20112302120027, Natural Scientific Research Innovation Foundation in Harbin Institute of Technology under Grant No. HIT.NSRIF.2014017, the Twelfth Government Advanced Research Fund under Grant No. 51317040302, 9140A17050114HT01054.

REFERENCES

- Q. Li, X.S. Zhou, and P. Lin, “Anomaly detection and fault diagnosis technology of spacecraft based on telemetry-mining,” 2010 3rd IEEE International Symposium on Systems and Control in Aeronautics and Astronautics (ISSCAA), 2010, pp. 233-236.

- K. Yamanishi, and Y. Maruyama, “Dynamic syslog mining for network failure monitoring,” Proceedings of the eleventh ACM SIGKDD international conference on Knowledge discovery in data mining, 2005, pp. 499-508.

- A. Zolghadr, “Advanced model-based fdir techniques for aerospace systems: Today challenges and opportunities,” Progress in Aerospace Sciences, vol.53, pp. 18-29, 2012.

- I. Hwang, S. Kim, and Y. Kim, “A survey of fault detection, isolation, and reconfiguration methods,” IEEE Transactions on Control Systems Technology, vol. 18, no. 3, pp.636-653, 2010.

- H. Wang, T. Chai, J. Ding, and M. Brown, “Data driven fault diagnosis and fault tolerant control: some advances and possible new directions,” Acta Automatica Sinica, vol. 35, no. 6, pp. 739-747, 2009.

- M. Daigle, X. Koutsoukos, and G. Biswas, “Improving diagnosability of hybrid systems through active diagnosis,” 2009 7th IFAC Symposium on Fault Detection, Supervision and Safety of Technical Processes (SAFEPROCESS 2009), Brussels, Belgium, 2009.

- A. Widodo, and B.S. Yang, “Support vector machine in machine condition monitoring and fault diagnosis,” Mechanical Systems and Signal Processing, vol. 21, no. 6, pp. 2560-2574, 2007.

- V. Muralidharan and V. Sugumaran, “A comparative study of Naïve Bayes classifier and Bayes net classifier for fault diagnosis of monoblock centrifugal pump using wavelet analysis,” Applied Soft Computing, vol. 12, no.8, pp. 2023-2029, 2012.

- J. Korbicz eds, “Fault diagnosis: models, artificial intelligence, applications,” Springer Science & Business Media, 2012.

- S. Li and P. Zhang, “Hydraulic system fault diagnosis method based on GA-PLS and AdaBoost,” Journal of Applied Statistics and Management, vol. 31, no.1, pp. 26-31, 2012.

- Q. Hu, Z. He, Z. Zhang and Y. Zi, “Fault diagnosis of rotating machinery based on improved wavelet package transform and SVMs ensemble,” Mechanical Systems and Signal Processing, vol.21, no. 2, pp. 688-705, 2007.

- M.A. El-Gamal, M. Mohamed, “Ensembles of neural networks for fault diagnosis in analog circuits,” Journal of Electronic Testing, vol. 23, no.4, pp. 323-339, 2007.

- A. Jordan, “On discriminative vs. generative classifiers: A comparison of logistic regression and naive bayes,” Advances in Neural Information Processing Systems, vol. 14, pp. 841, 2002.

- L.I. Kuncheva, M. Skurichina, and R. Duin, “An experimental study on diversity for bagging and boosting with linear classifiers”, Information Fusion, vol.3, no.4, pp. 245-258, 2002.

- Y. Freund and R. Schapire, “Experiments with a new boosting algorithm” Machine Learning: Proceedings of the Thirteenth International Conference, 1996, pp. 148-156.

- K Goebel, The Dataset of Maintenance Action Recommendation, https://c3.nasa.gov/dashlink/resources/842/

- V Katsouros, V Papavassiliou and C Emmanouilidis, “A Bayesian Approach for Maintenance Action Recommendation,” International Journal of Prognostics and Health Management, vol.4, no.2, pp. 1-6, 2013.

Author Biography

He Sijie was born in Inner Mongolia, China, in 1993. She received B.S. from Harbin Institute of Technology (HIT), Harbin, China, in 2014. Now she is a master candidate with the Department of Automatic Test and Control, HIT. Her research interests include data analysis and mining, fault diagnosis for aerospace applications.

Peng Yu (M’ 10) received the B.S. degree in Harbin Institute of Technology (HIT), Harbin, China, in 1996, and M.Sc. and Ph.D. degrees in HIT, in 1998 and 2004, respectively. He is a full Professor with the Department of Automatic Test and Control, HIT. He is also the vice dean of School of Electrical Engineering and Automation, HIT. His research interests include automatic test technologies, system health management, reconfigurable computing, etc.

Liu Datong (M’ 11) received his B.Sc. degree in Harbin Institute of Technology (HIT), Harbin, China, in 2003. During 2001 to 2003, he also minored in the Department of Computer Science and Technology at HIT. He received the M.Sc. and Ph.D. degrees in HIT in 2005 and 2010, respectively. He is currently an associate professor in the Department of Automatic Test and Control, HIT. His research interests include automatic test, test data processing of complex system, data-driven prognostics and health management, lithium-ion battery prognostics, system health management for aerospace, etc.